Laboratorium Dasar Teknik Elektro – STEI ITB
	Nama Kuliah (Kode)     	: Praktikum Sistem Mikroprosesor (EL3112)

Tahun / Semester	: 2026-2027 / Ganjil

Modul	:  1

Nama Asisten / NIM	:

Nama Praktikan/NIM	: Muhammad Aqeel Ghani / 13224071

Tugas Pendahuluan

- Instalasi Arduino IDE

- Instalasi PlatformIO di Visual Studio Code

- Mencoba Compile contoh-contoh kode yang diberikan

- Output Digital dengan akses register ESP32

#include "soc/gpio_reg.h" #include "driver/gpio.h" #include <Arduino.h> #define LED_PIN 2 // Built-in LED on most ESP32 boards #define BLINK_DELAY_MS 500 // 500ms on + 500ms off = 1Hz void setup() { // Configure GPIO pin as output using register access // Enable the GPIO pin in the output enable register REG_WRITE(GPIO_ENABLE_REG, REG_READ(GPIO_ENABLE_REG) ¦ (1 << LED_PIN)); // Set pin function to GPIO (not special function) PIN_FUNC_SELECT(GPIO_PIN_MUX_REG[LED_PIN], PIN_FUNC_GPIO); // // Optional: Set pin to push-pull mode (default, but explicit) // REG_SET_BIT(GPIO_PIN_REG[LED_PIN], GPIO_PIN_PAD_DRIVER); Serial.begin(115200); Serial.println("ESP32 Register-based LED Blink Started"); } void loop() { // Turn LED ON - Set the bit high using register access REG_WRITE(GPIO_OUT_REG, REG_READ(GPIO_OUT_REG) ¦ (1 << LED_PIN)); Serial.println("LED ON"); delay(BLINK_DELAY_MS); // Turn LED OFF - Clear the bit using register access REG_WRITE(GPIO_OUT_REG, REG_READ(GPIO_OUT_REG) & ~(1 << LED_PIN)); Serial.println("LED OFF"); delay(BLINK_DELAY_MS); }

- Mencatat MAC Address (Identitas) ESP32

#include <WiFi.h> #include <esp_wifi.h> void readMacAddress(){ uint8_t baseMac[6]; esp_err_t ret = esp_wifi_get_mac(WIFI_IF_STA, baseMac); if (ret == ESP_OK) { Serial.printf("%02x:%02x:%02x:%02x:%02x:%02x\n", baseMac[0], baseMac[1], baseMac[2], baseMac[3], baseMac[4], baseMac[5]); } else { Serial.println("Failed to read MAC address"); } } void setup(){ Serial.begin(115200); WiFi.mode(WIFI_STA); Serial.print("[DEFAULT] ESP32 Board MAC Address: "); readMacAddress(); } void loop(){ }

- Mengukur Kecepatan Output Digital dengan Library Arduino

#include <Arduino.h> #define LED_BUILTIN 22 void setup(){ pinMode(LED_BUILTIN, OUTPUT); } void lopp(){ digitalWrite(LED_BUILTIN, HIGH); digitalWrite(LED_BUILTIN, LOW); }

- Mengukur Kecepatan Output Digital dengan Akses Register

- Latency Input Output Digital dengan Library Arduino

- Mempersiapkan tugas-tugas kode yang diberikan

- Latency Input Output Digital dengan Akses Register

- Pengendali Relay

- Pengukur Frekuensi

- Menyiapkan tabel-tabel pengukuran di setiap percobaan